The main objective of the script is to evaluate the technical impact of integrating a Photo-Voltaic (PV) system into the grid by comparing two distinct scenarios over a 2,160-hour period (which equals 90 days or one fiscal quarter of hourly data):

- Case 1: Before PV integration (Baseline).

- Case 2: After PV integration (With PVSystem.PV675 enabled).

Here is a detailed breakdown of what the code accomplishes step-by-step:

## 1. Environment Initialization & OpenDSS Connection

- Clears the MATLAB workspace, command window, and closes any open plots.

- Establishes a COM interface link to the OpenDSS engine (actxserver('OpenDSSEngine.DSS')).

- Compiles the OpenDSS network model file located at D:\MINIPROJECT\MINIPROJECT\IEEE13Master1.dss.

- Dynamically extracts the names of all electrical buses in the circuit.

## 2. Iterative Simulation Loops (2,160 Hours)

The script runs two separate, massive loops—one for Before PV and one for After PV. For every single hour, it:

- Moves the simulation clock forward (DSSSolution.Hour = hr-1).

- Solves the power flow snapshot (DSSSolution.Solve).

- Extracts Losses: Records total active ($kW$) and reactive ($kVAR$) technical line losses.

- Extracts Node Voltages: Iterates through every bus, identifies the active phases, and records the voltage magnitudes in both Line-to-Neutral ($V$) and Per-Unit ($PU$) formats.

## 3. Data Processing & Comparative Analytics

Once both loops finish, the script aggregates the hourly data to compute:

- Total integrated active energy losses ($kWh$) and reactive energy losses ($kVARh$) for both scenarios.

- The overall net reduction in energy losses achieved by adding the solar array.

- A summary comparison table (LossComparison) displayed directly in the MATLAB Command Window.

## 4. Visualizations (Matplotlib-style Plots)

The code generates four distinct graphical plots to visualize the grid behavior:

- Active Power Loss Profile: A 2,160-hour timeline showing how active losses ($kW$) fluctuate and drop after PV installation.

- Reactive Power Loss Profile: A timeline tracking reactive power loss ($kVAR$) trends.

- Final Hour Voltage Profile: A detailed node-by-node snapshot comparing the Per-Unit voltage across every phase/bus index specifically at the final hour (Hour 2160).

- Minimum Voltage Horizon: A timeline tracking the absolute worst-case (lowest) voltage present on the feeder during every hour, showing if the PV system helps prevent low-voltage violations.

## 5. Data Export & Archiving

Finally, the script automates data reporting by saving the results across multiple formats:

- Yearly_Losses_Comparison.xlsx: Excel sheet containing hourly data for both active and reactive losses.

- Total_Yearly_Loss_Comparison.xlsx: Excel summary of total accumulated energy losses.

- FinalHourVoltageProfile.xlsx: Excel sheet documenting individual bus-phase voltage data for the 2,160th hour.

- IEEE13_YearlyResults.mat: Saves all generated MATLAB workspace variables into a binary file for future reload or deep-dive analysis.